Commisioning Report

For Site:

<aggiungere nome ABN e sito>

_______________________				_____<nome e societa collaudatore>____

Vodafone Representative:					Field partner representative:

Date: __________________				Date: __________________

Version 1.7

Commissioning Date:

Attendee:

Revision history:

| No | Item | Author | Date |
| --- | --- | --- | --- |
| 1.0 | First release | F. Vismara | 02/07/2020 |
| 1.1 | Add. sec 6.3 and 6.4 | F. Vismara | 25/09/2020 |
| 1.2 | Updated to V3R7C00SPC100 | F. Vismara | 05/10/2021 |
| 1.3 | IMPORTANT – Updated sec. 6.2 | F. Vismara | 14/06/2022 |
| 1.4 | Updated to V8R21C00SPC100 | F. Vismara | 05/08/2022 |
| 1.5 | Add. new SFP 1G and 10G BOM (pag. 6) | F. Vismara | 20/09/2022 |
| 1.6 | Add. new SFP 1G and 10G BOM (pag. 6) Patch upgraded to V800R021SPH180 | L. Angarella | 06/02/2023 |
| 1.7 | Patch upgraded to HP2201 | F. Vismara | 25/08/2023 |

Table of Contents

1 Materials Needed (p. 3 Commissioning Procedure)	4

2 Device Photo (p. 3)	4

3 Software & License Check (p. 4)	4

4 Equipment Health Check (p. 4)	4

4.1 Disks Space	4

4.2	Memory and CPU Usage	4

4.3	Check Temperature	4

4.4	Check Fans	4

5 Power Check (p. 5)	5

5.1 Power Redundancy Check	5

5.2 Voltage Check	5

6 Ports Test (p. 5)	5

6.1 Modules Check	5

6.2 BOM Check	6

6.4 Port License Resource Distribution Check	6

7 Collect Device Information (p. 8)	6

8 Configuration Check (p. 8)	7

9 Alarms Check (p. 8)	7

9.1 Alarm Active / All	7

9.2 Alarm History	7

10 Check GPS Signal (p. 9)	7

11 Non Standalone checks (p. 10)	7

11.1 LLDP Neighbor Check and interface check	7

- Materials Needed (p. 3 Commissioning Procedure)

- Prima di iniziare, assicurarsi di avere tutti i materiali necessari.

- Device Photo (p. 3)

- Riportare qui una fotografia dell’ATN a lavoro terminato.

- Software & License Check (p. 4)

Riportare qui l’output dei comandi DISPLAY VERSION e DISPLAY LICENSE - Risultato atteso: Software version ATN910C-G V800R021C00SPC100 - Risultato atteso: Patch version V800R021HP2201

- Equipment Health Check (p. 4)

- 4.1 Disks Space

Riportare qui l’output del comando DIR /ALL-FILESYSTEMS Risultato atteso: NIENTE FILE .CC o .PAT NON CONFORMI

  - Memory and CPU Usage

Riportare qui l’output del comando DISPLAY HEALTH

  - Check Temperature

Riportare qui l’output del comando DISPLAY TEMPERATURE

  - Check Fans

Riportare qui l’output del comando DISPLAY FAN

- Power Check (p. 5)

- 5.1 Power Redundancy Check

Riportare qui l’output dei comandi DISPLAY DEVICE e DISPLAY POWER

- 5.2 Voltage Check

Riportare qui l’output del comando DISPLAY VOLTAGE

- Ports Test (p. 5)

The received optical power is measured using a loop inserted on ODF side.

The Optical modules transmitting power reference value as below:

1GE Transmitting power	:  -9.0 dBm < Tx Power < -3.0d Bm

10GE Transmitting power	:  -8.0 dBm < Tx Power < -1.0 dBm

>>> ESEGUIRE PRIMA

sys

port-group group-member G0/2/1 to G0/2/19

undo shutdown

commit

- 6.1 Modules Check

Riportare qui l’output del comando DISPLAY OPTICAL-MODULE BRIEF

>>> POI ESEGUIRE

rollback configuration last 1

y

PRIMA DI PROCEDERE CON IL TEST SUCCESSIVO

IMPORTANTE – I test seguenti DEVONO essere eseguiti SOLO con i moduli ottici che fanno parte della dotazione dell’apparato.

- 6.2 BOM Check

Riportare qui l’output dei comandi dis elabel ¦ in Item=3406 e dis elabel ¦ in Item=3406 ¦ ex 0742¦0742-00¦0599¦0672¦0672-001¦0687¦0473¦0476¦0483¦2523¦0286 ¦ count Risultato atteso x 10GB – 34060742 / 34060742-00x / 34060599 / 34060672 / 34060672-001 / 34060687 Risultato atteso x 1GB – 34060473 / 34060476 / 34060483 / 34062523 / 34060286 Risultato atteso - Total lines: 0

6.3 Port Bandwidth Check

Riportare qui l’output del comando display interface brief: Risultato atteso – NESSUNA PORTA DEVE SEGNARE(100M)

- 6.4 Port License Resource Distribution Check

Riportare qui l’output del comando display license resource usage port-basic all: Risultato atteso – NESSUNA PORTA DEVE ESSERE IN STATO Allocated

- Collect Device Information (p. 8)

Riportare qui l’output del comandi: DISPLAY ELABEL BRIEF DISPLAY ELABEL DISPLAY ELABEL BACKPLANE DISPLAY ELABEL OPTICAL-MODULE BRIEF Codice Item atteso x board slot 2: 02352YTQ or 02352YTQ-003 Codice BarCode atteso x modulo GPS 905M-A: 034CGQXXXXXXXXXX

- Configuration Check (p. 8)

SE LA CONFIG DEFINITIVA E’ DISPONIBILE, riportare qui l’output del comando DISPLAY CONFIGURATION RECOVER-RESULT. ALTRIMENTI SCRIVERE “CFG NON DISPONIBILE”. - Risultato atteso: The number of failed commands is 0

- Alarms Check (p. 9)

- 9.1 Alarm Active / All

Riportare qui l’output del comandi DISPLAY ALARM ACTIVE e DISPLAY ALARM ALL - Risultato atteso ALARM ACTIVE: Eth-Trunk3001 - Risultato atteso ALARM ALL: nessun allarme

- 9.2 Alarm History

Riportare qui l’output del comando DISPLAY ALARM HISTORY - Risultato atteso: nessun allarme

- Check GPS Signal (p. 10)

SOLO IN CASO DI PORTA CONFIGURATA E ATTIVA, riportare qui l’output del comando DISPLAY SMART-CLOCK INTERFACE G0/2/27. ALTRIMENTI SCRIVERE “GPS NOT CONFIGURED”. - Risultato atteso: Time lock success :yes Freq lock success :yes

- Non Standalone checks (p. 10)

  - LLDP Neighbor Check and interface check

Riportare qui l’output dei comandi: DISPLAY LLDP NEIGHBOR BRIEF DISPLAY INTERFACE G0/2/0 NB: da effettuarsi solo dopo cablaggio della porta 0/2/0 verso Optix.